# AVENIR Pillar 2 Catalogue

# EUROCONTROL Tools: Fuel Burn Scope and User Mapping

[as discussed today, we should use Catalogue website to publish information about the fuel burn calculation difference and the needs coming from the Asu tools and data.]

Several EUROCONTROL ASU tools and methodologies provide fuel burn–related outputs. These outputs are sometimes interpreted as being directly comparable or interchangeable, which is not the case. Differences primarily arise from scope, aggregation level, phase-of-flight coverage, and intended users. The mapping below is therefore explanatory, not prescriptive.

This note provides a focused explanation of how selected EUROCONTROL tools address fuel burn calculations, how these calculations differ in scope and purpose, and how tools are mapped to different users and use contexts. The mapping is illustrative and explanatory only; it does not rank tools, validate indicators, or imply equivalence.

| Tool | Primary users | Typical fuel burn scope | Spatial / aggregation level | Phase-of-flight coverage | Typical use context |
| --- | --- | --- | --- | --- | --- |
| AEM (Advanced Emissions Model) | Analysts, researchers, ANSPs, policy support | Physics-based estimation of fuel burn and emissions | Flight / trajectory level, scalable to aggregates | Supports gate-to-gate | Methodological analysis, emissions modelling, research and policy support |
| SET (Small Emitters Tool) | Aircraft operators (small emitters); verifiers/administering States (EU/UK/Swiss ETS context) | Statistical estimation of fuel burn and associated CO₂ emissions for flights within ETS reporting scope (statistical model) | Flight list / operator fleet level (supports totals over lists of flights) | Entire flight as represented by the tool (based on statistical models and flight characteristics) | ETS monitoring and reporting support; data-gap reporting; simplified monitoring for eligible operators |
| IMPACT | Network Manager, analysts | Consistent network-wide fuel burn and emissions estimates | Pan-European network level | Typically airborne | Network performance monitoring, reporting, trend analysis |
| FlyingGreen | ANSPs, States, airports, Policy makers (module-dependent) | Fuel burn and emissions indicators derived from multiple data sources (using AEM) | Configurable: ANSP, airport, State, or network views | Phase-dependent, module-specific | Transparency, stakeholder dialogue, awareness, decision support |

The tools and platforms listed differ in nature (model, tool, platform) and role; the mapping reflects typical use and scope and does not imply equivalence or overlap in mandate.

## Caveats

- Fuel burn values from different tools should not be directly compared without understanding scope and assumptions.

- No single tool provides a complete representation of ANSP environmental performance.

- Interpretation must always reflect the intended user and decision context.